SUMMER INTERNSHIP Embedded C

# - K Lokeshwar Reddy- BU21EECE0100334 - lkondapp@gitam.in

TASK 12

## Q1)

#include <stdio.h> int countSetBits(int n) {     int count = 0;     while (n) {         count += n & 1;         n >>= 1;

}

return count;

}

int main() {     int num;     prin ("Enter an integer: ");     scanf("%d", &num);     int setBits = countSetBits(num);     prin ("Number of set bits in %d is %d\n", num, setBits);

return 0;

}

## Q2)

#include <stdio.h> int main()

{

unsigned short int value = 0;     unsigned short int mask = (1 << 4) | (1 << 11);     value |= mask;     prin ("The value a er se	ng the 5th and 12th bits is: %u\n", value);

return 0;

}

## Q3)

#include <stdio.h> unsigned int clearBits(unsigned int num) {     unsigned int mask = ~((1 << 5) | (1 << 18));      return num & mask;

}

int main() {     unsigned int num;     prin ("Enter a 32-bit unsigned integer: ");     scanf("%u", &num);     unsigned int result = clearBits(num);     prin ("Result a er clearing the 6th and 19th bits: %u\n", result);

return 0;

}

## Q4)

#include <stdio.h> unsigned short flipEvenBits(unsigned short num) {     unsigned short mask = 0x5555;      return num ^ mask;

}

int main() {     unsigned short num;     prin ("Enter a 16-bit unsigned integer: ");     scanf("%hu", &num);     unsigned short result = flipEvenBits(num);     prin ("Result a er flipping the even-posi oned bits: %hu\n", result);

return 0;

}

## Q5)

#include <stdio.h> unsigned int packIP(unsigned char a, unsigned char b, unsigned char c, unsigned char d) {     return (a << 24) | (b << 16) | (c << 8) | d;

}

int main() {     unsigned char a = 192;     unsigned char b = 168;     unsigned char c = 1;     unsigned char d = 100;     unsigned int packedIP = packIP(a, b, c, d);     prin ("Packed IP address: 0x%X\n", packedIP);

return 0;

}

## Q6)

#include <stdio.h> int main() {     unsigned int packed_ip = 0xC0A80164;     unsigned char a = (packed_ip >> 24) & 0xFF;      unsigned char b = (packed_ip >> 16) & 0xFF;      unsigned char c = (packed_ip >> 8) & 0xFF;       unsigned char d = packed_ip & 0xFF;             prin ("The unpacked IP address is: %u.%u.%u.%u\n", a, b, c, d);     return 0;

}

## Q7)

#include <stdio.h> #include <stdlib.h> unsigned long long convertMACAddress(const char *mac) {

unsigned int bytes[6];

if (sscanf(mac, "%x:%x:%x:%x:%x:%x", &bytes[0], &bytes[1], &bytes[2], &bytes[3], &bytes[4],

&bytes[5]) != 6) {         fprin (stderr, "Invalid MAC address format.\n");         exit(EXIT_FAILURE);

}

unsigned long long macBinary = 0;

for (int i = 0; i < 6; ++i) {

macBinary = (macBinary << 8) | (bytes[i] & 0xFF);

}

return macBinary;

}

int main() {     char macString[18];     prin ("Enter MAC address in the format XX:XX:XX:XX:XX:XX: ");     if (scanf("%17s", macString) != 1) {         fprin (stderr, "Failed to read MAC address.\n");         return EXIT_FAILURE;

}

unsigned long long macBinary = convertMACAddress(macString);     prin ("MAC address in 48-bit binary pa ern: %012llx\n", macBinary);

return 0;

}

## Q8)

#include <stdio.h> #include <stdlib.h> void binaryToMac(const char* binary) {

unsigned int bytes[6] = {0};     for (int i = 0; i < 48; ++i) {         bytes[i / 8] = (bytes[i / 8] << 1) | (binary[i] - '0');      }

prin ("MAC Address: %02X:%02X:%02X:%02X:%02X:%02X\n",

bytes[0], bytes[1], bytes[2], bytes[3], bytes[4], bytes[5]);

}

int main() {

const char* binary_pa ern = "101010101011101111001100110111011110111111111111";     binaryToMac(binary_pa ern);

return 0;

}

# Task 14

## 1)bare metal blinky using arduino1

#define F_CPU 16000000UL

#include <avr/io.h>        #include <u l/delay.h>    int main(void)

{

// Set pin 7 (PD7) as an output     DDRD |= (1 << PD7);

while (1)

{

PORTD |= (1 << PD7);

_delay_ms(1000);

PORTD &= ~(1 << PD7);

_delay_ms(1000);

}

return 0;

}

## 2)bare metal push bu on1

#define F_CPU 16000000UL

#include <avr/io.h>        #include <u l/delay.h>    const uint8_t bu onPin = PD2;      const uint8_t ledPin = PB5;

uint8_t bu onState = 0; void setup() {

DDRD &= ~(1 << bu onPin);

PORTD |= (1 << bu onPin);

DDRB |= (1 << ledPin);

}

int main(void) {

setup();     while (1) {         bu onState = PIND & (1 << bu onPin);

if (bu onState) {

PORTB |= (1 << ledPin);

} else {

PORTB &= ~(1 << ledPin);

}

_delay_ms(10);

}

return 0;

}

# Task 15

## Analog Read (Poten ometer)

// Constants for pin assignments

const int potPin = A0;    // Pin where the poten ometer is connected

const int ledPin = 9;     // Pin where the LED is connected

// Variable to store the poten ometer value int potValue = 0;

void setup() {

// Ini alize the LED pin as an output   pinMode(ledPin, OUTPUT);

}

void loop() {

// Read the value from the poten ometer   potValue = analogRead(potPin);

// Map the poten ometer value to the PWM range (0-255)   int ledValue = map(potValue, 0, 1023, 0, 255);

// Set the brightness of the LED   analogWrite(ledPin, ledValue);

// Small delay to smooth out the reading   delay(10);

}

## Analout Output(fading)

const int ledPin = 9;      // Pin where the LED is connected

void setup() {

// Ini alize the LED pin as an output   pinMode(ledPin, OUTPUT);

}

void loop() {   // Fade in from 0 to 100^6   for (int brightness = 0; brightness <= 100^6; brightness++) {     analogWrite(ledPin, brightness);  // Set the brightness     delay(10);  // Wait for 10 milliseconds

}

// Fade out from 100^6 to 0   for (int brightness = 100^6; brightness >= 0; brightness--) {     analogWrite(ledPin, brightness);  // Set the brightness     delay(10);  // Wait for 10 milliseconds

}

}

## Digital Input using Interrupt

const int bu onPin = 2;    // Pin where the push bu on is connected vola le bool bu onPressed = false; // Flag to indicate bu on press void setup() {   pinMode(bu onPin, INPUT);          // Set the bu on pin as input

a achInterrupt(digitalPinToInterrupt(bu onPin), bu onPressISR, RISING); // A ach interrupt on rising edge

Serial.begin(9600);                 // Ini alize serial communica on

}

void loop() {   if (bu onPressed) {

Serial.println("Bu on Pressed!");  // Print message when bu on is pressed

bu onPressed = false;              // Reset the flag

}

}

void bu onPressISR() {   bu onPressed = true; // Set the flag to indicate bu on press

}